# Outline of HAL-1 Documentation

## Overview

HAL-1 is the Liquid Rocketry at Illinois (LRI) avionics board for the 2025-2026 academic year. It accomplishes the tasks of logging and managing:

- Altitude data

- Positional data (including but not limited to Euler angles, velocity, and acceleration)

- Testing of modular redundancy systems and procedures

The project also aims to establish many critical logistical and technical procedures, including:

- Standardized project workflow and organization

- Creating a framework for future boards based on STM32 system architecture

- Easing the development process of avionics for future generations of LRI

## Hardware Components and Procedures

This section outlines the components used in HAL-1 and includes links to datasheets and external references. The goal of this section is to introduce the reader to the hardware being used, and explain topics such as communication protocols, datasheet information extraction, and how decisions for components were made.

### Microcontroller (MCU): STM32h750VBT6

The STM32h750VB series is a robust line of STM32 microcontrollers, and are based on a 32 bit Arm architecture. The VBT6 LQFP100 was chosen for use, with 100 pins on the chip. This model is powerful enough to handle computation for all sensors’ data, and is a viable choice of microcontroller for the project. Some notable features include:

- 480 MHz clock speed

- 128 KBytes flash memory

- 1 Mbyte of RAM

- 3.3V logic (1.62V – 3.6V application supply voltage)

- Voltage supplied to VBAT (Backup voltage) must be above 1.55V

#### V_DD, V_BAT, V_DDA

These three voltage pins are inputs. They accept the 3.3V in use for all modules on HAL-1. The difference between V_DD and V_DDA is that V_DDA is a separate voltage supply for the analog components within the chip itself. The usage of the V_DDA pin is usually to allow a filtered input to be supplied but is not implemented in HAL-1 in the interest of simplicity and quick functionality. V_BAT is the backup power source, preferably from a 3.3 battery source, although the MCU can accept anything from 1.62V to 3.6V. If no backup battery exists, as is the case for HAL-1, this pin may be connected to 3.3V power. Regardless of usage, V_DD, V_BAT, and V_DDA must all be connected to power.

### Inertial Measurement Unit (IMU): BNO085

### Barometer: BMP390

### Temperature and Humidity sensor: SHT41

### GPS: ublox MAX-M8Q

### Radio: Ebyte E22 900T22S

## STM32 CubeMX and MCU Pinout

Shdjfg

## KiCAD and Hardware Project Organization